REVOLT SYSTEM

Remote Energy Via Optimized Load Turbine

Technical Brief — Makers, Manufacturers, NGOs, and Humanitarian Organizations

| Document Type | Originator | Disclosure Date | License |
| --- | --- | --- | --- |
| Technical Brief — Open Source | [Your Name] | March 3, 2026 | CC0 — Public Domain |

All designs, code, and specifications in this disclosure are released to the public domain. Any individual, organization, or manufacturer may produce, distribute, or modify this system without restriction or royalty.

# Executive Summary

REVOLT is a fully 3D-printable modular Vertical Axis Wind Turbine (VAWT) with an integrated axial flux generator, designed specifically for emergency, humanitarian, and off-grid deployment contexts. The design uses only globally available commodity hardware — standard skateboard bearings, hardware store rod stock, and sub-$5 electronic modules — making it manufacturable anywhere in the world with access to a basic FDM 3D printer.

| Design Goal | Solution | Outcome |
| --- | --- | --- |
| Zero special tooling | All structural parts FDM printed, hardware bin parts | Buildable in any country with a printer |
| Scalable output | Modular snap-fit tier stacking | Add tiers for more power without redesign |
| Zero-wind capability | Hand crank overdrive on same shaft interface | Unconditional emergency power source |
| Multi-surface deployment | Universal mount — PVC, T-post, vehicle, tripod | Single design for all deployment contexts |
| Community validation | Truck-bed wind tunnel protocol + fatigue logs | Crowd-sourced performance and durability data |

# System Architecture

## Rotor — Savonius Drag Design

The Savonius configuration is selected over lift-based designs (Darrieus, H-rotor) for the following reasons relevant to emergency deployment:

- Starts rotating at wind speeds as low as 2-3 mph — critical in low-wind shelter environments

- Omnidirectional — no yaw mechanism required, reducing complexity and failure points

- Self-limiting RPM in high winds — reduces overspeed risk without active governors

- Geometry is entirely printable without support structures when oriented correctly

## Venting Gap Geometry

The critical performance parameter is the overlap ratio — the offset between the two scoop centerlines at the hub:

overlap_offset = rotor_diameter * 0.18   // 18% recommended

scoop_center_x = (R/2) - (overlap/2)    // scoop mounting offset from axis

At 18% overlap, air captured by the power-stroke scoop bleeds through the center gap and applies a secondary positive pressure on the returning scoop's convex face. This reduces net drag on the returning stroke by approximately 15-20% compared to a zero-overlap design, directly increasing the power coefficient Cp.

## Performance Model

Power output is modeled using the standard wind power equation with empirical Cp for Savonius rotors:

P = Cp * (0.5 * rho * A * v^3)

A = rotor_diameter * rotor_height    // swept frontal area

Cp = 0.22  (18% overlap Savonius, empirically validated range 0.20-0.25)

rho = 1.225 kg/m3  (sea level, 15 deg C)

| Wind (mph) | Wind (m/s) | RPM (single tier) | W per Tier | W — 3 Tiers | W — 5 Tiers |
| --- | --- | --- | --- | --- | --- |
| 10 | 4.47 | 427 | 0.96 | 2.9 | 4.8 |
| 15 | 6.71 | 640 | 3.25 | 9.8 | 16.3 |
| 20 | 8.94 | 854 | 7.71 | 23.1 | 38.5 |
| 25 | 11.18 | 1067 | 15.05 | 45.1 | 75.2 |
| 30 | 13.41 | 1281 | 26.00 | 78.0 | 130.0 |

# Generator Specification

## Axial Flux Architecture

The generator uses an axial flux topology chosen for printability and low-RPM efficiency. Both rotor (magnet) and stator (coil) discs are 3D-printed polymer with embedded components. The axial gap is nominally 1.5-2.0mm, adjustable via shaft collar position.

## Coil Winding — 5V USB Target at 300 RPM

| Parameter | Value | Derivation |
| --- | --- | --- |
| Target output | 5V DC, 300 RPM | Emergency USB charging threshold |
| Pole pairs | 4 (8 magnets total) | N52, 22mm dia, alternating N-S |
| Electrical frequency at 300 RPM | 20 Hz | f = (RPM/60) * pole_pairs |
| Required peak AC voltage | 7.07V | V_DC * sqrt(2) |
| Flux per pole (Phi) | ~0.0008 Wb | N52 magnet, 2mm air gap |
| Turns per coil | 100 turns | N = Vpeak / (4.44 * f * Phi) |
| Coil count | 9 coils | 3-phase, 3 coils per phase |
| Wire gauge | 24 AWG magnet wire | Balance of resistance and fill |
| Phase connection | Star (Y) | Neutral center point connected |

## Output Chain

| Stage | Component | Spec | Cost (approx) |
| --- | --- | --- | --- |
| Rectification | 3-phase bridge rectifier module | Standard 3-phase, 50V/10A rated | ~$2 |
| Smoothing (optional) | Supercapacitor bank | 10-50F, handles gust variation | ~$5-15 |
| USB output | 5V/2A buck converter module | XL4016 or equivalent | ~$2 |
| Battery charging | PWM or MPPT charge controller | 12V or 24V, 10A minimum | ~$8-20 |

# Bill of Materials

## 3D Printed Components

| Part | Qty per Tier | Material | Est. Filament |
| --- | --- | --- | --- |
| Savonius scoop blade | 2 | PETG or ASA | ~80g each |
| Hub collar | 1 | PETG or ASA | ~40g |
| Base mount | 1 (total) | PETG or ASA | ~120g |
| Generator rotor disc | 1 (total) | PETG or ASA | ~60g |
| Generator stator disc | 1 (total) | PETG or ASA | ~60g |
| Hand crank arm | 1 (optional) | PETG or ASA | ~50g |

## Hardware (Bin Parts — Globally Available)

| Part | Spec | Qty | Approx Cost |
| --- | --- | --- | --- |
| Central shaft | 12.7mm (1/2") SS or aluminum rod | 1 — cut to height | $3-8 |
| Bearings | 608-RS (skateboard bearing) | 2 | $2-4 |
| Neodymium magnets | N52, 22mm dia x 6mm thick | 8 | $8-15 |
| Magnet wire | 24 AWG, 1 spool | 1 | $5-10 |
| Bridge rectifier | 3-phase module | 1 | $2 |
| Buck converter | 5V/2A module | 1 | $2 |
| M5 bolts | 20mm length | 4 | $1 |

Total estimated build cost (3-tier, single USB output): $30-60 USD depending on location and existing filament stock.

# Deployment Contexts

| Context | Configuration | Key Features Used | Expected Output |
| --- | --- | --- | --- |
| Disaster relief camp | 3-5 tier, PVC pole mount | USB charging + 12V battery | Comms and lighting for ~10 people |
| Remote village — no grid | 8 tier, permanent post | 12V/24V battery bank | Sustained off-grid power |
| Military/field use | 3 tier, vehicle roof rack | USB + hand crank backup | Device charging, mobile |
| Individual emergency kit | 1-2 tier, tripod | USB only + hand crank | Phone and radio charging |
| Maker/education | Any config | Truck-bed validation protocol | Learning + data contribution |

# Community Data Loop — Fatigue Logs

The repository includes a standardized Fatigue Log template. Builders are encouraged to upload:

- Material used and print settings

- Tier count and rotor diameter

- Wind environment (average and peak speeds)

- Hours of operation before first component failure

- Failure mode description (crack location, delamination, bearing wear, etc.)

- Truck-bed test power curves if conducted

This crowd-sourced dataset will drive V2.0 geometry improvements, particularly for wall thickness optimization and hub stress concentration points.

# Repository Contents

| File | Description |
| --- | --- |
| revolt_scoop.scad | Full OpenSCAD parametric source — all components |
| revolt_simulation.py | Physics simulation — power/torque curves at any wind speed |
| REVOLT_Plain_English.pdf | Non-technical guide for general builders |
| REVOLT_Patent_Disclosure.pdf | Full prior art disclosure establishing public domain |
| REVOLT_Technical_Brief.pdf | This document |
| fatigue_log_template.csv | Standardized community data upload format |
| DISCLOSURE_DATE.txt | Timestamped prior art record |
| README.md | Project overview and build guide |

# Closing Statement

REVOLT is released with no strings attached under CC0. Print it, build it, sell kits, deploy it in disaster zones, teach it in schools. The only goal is that people have power when they need it most.

| Contact | Repository | Hashtags |
| --- | --- | --- |
| X: @OpnSorcePatents | github.com/OpenSourcePatents/REVOLT | #OpenSourceEnergy #REVOLT #OpenSourceSafety |

REVOLT System  |  Remote Energy Via Optimized Load Turbine  |  Open Source Energy Architecture